Lab#8 – Lecture 4d

Names: ____________________, ____________________

Date: _____________________

Objective:

This lab #8 is intended to introduce the students to be more comfortable of using the Lab Equipment:

The logic designer, the test equipment, and of course the TTL chips. Most of all you’ll learn more about Design Logic Circuits.

Procedure:

Use an existing Problem 4-23 on page 225 as show above to perform the following:

- Use Multisim  to draw an XOR logic circuit for a 74LS86 call Figure 1

- Simulate Figure 1 for each voltage level and record in Table 1.

- Implement on the breadboard for testing and measuring each voltage level.

- Record test result in Table 1

- Repeat step 1-4 using a logic gate NOT, AND, and OR (74LS05, 74LS08 and 74LS32)*

- Record test result for step 5 in Table 2

- Conclusion

- *

Select two 1kohm resistors.

Reminder:  Measure and record the resistance of each resistor.

Equipment needed:

1 – Power Supply

1 – Digital Multimeter

2– 1Kohm

1 – 4 position dip switch

1 – 74LS86N - XOR

2– 74LS05N- NOT

2 – 74LS08N- AND

1 – 74LS32N - OR

3 – LED (light Emitted Diode)

| Simulated (Input) |  | Result | Test (Input) |  |  | Result |
| --- | --- | --- | --- | --- | --- | --- |
| A | B |  | A |  | B |  |
| 0 | 0 |  |  |  |  |  |
| 0 | 1 |  |  |  |  |  |
| 1 | 0 |  |  |  |  |  |
| 1 | 1 |  |  |  |  |  |

Table 1 (Simulation vs Test)

Table 1- Lab#8 Schematic

| Simulated (Input) |  | Result | Test (Input) |  |  | Result |
| --- | --- | --- | --- | --- | --- | --- |
| A | B |  | A |  | B |  |
| 0 | 0 |  |  |  |  |  |
| 0 | 1 |  |  |  |  |  |
| 1 | 0 |  |  |  |  |  |
| 1 | 1 |  |  |  |  |  |

Table 2 XNOR (Simulation vs Test)

Observations:______________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________