Week 3

A capacitor is one of the basic electrical components. By definition, “A capacitor is an electrical component that stores electrical charge when voltage is applied.” A capacitor stores an electrical charge using of two conducting surfaces (often called capacitor plates) that are separated by an insulator called a dielectric.

A capacitor’s capacity is measured in a unit called Farads, named after physicist Michael Faraday for his work within the electrical industry. A farad is “The SI unit of electrical capacitance, equal to the capacitance of a capacitor in which one coulomb of charge causes a potential difference of one volt.” A capacitor’s capacity is determined by the size of the plates, the material of the plates, the material of the dielectric, and the distance between the two plates.

In a circuit, if a group of capacitors is wired in series, the total capacitance will be less than the lowest value capacitor. However, when connected in parallel, the total capacitance increases by the value of each added capacitor. Capacitors have many uses in circuits. Their charge and discharge rates are necessary for timing circuits. Their ability to absorb large amounts of energy is useful as a protection against surges.

There are many different types of capacitors, but they can be split into two different categories, polarized and non-polarized. All capacitors will bock DC voltage, but allow AC voltage to pass through. This property is used in AC circuits when DC is not wanted.