Week 5

An oscilloscope is a tool used to measure changes in voltage over a very short period of time. This allows you to test peak to peak voltage, high frequencies, and can view signals with extreme precision.  When taking any measurements, the probe will always have the ground clip connected to the circuit’s ground, and the probe will be connected to the point or component that you wish to take a measurement at. If the probe’s 10x switch is turned on, any measurements taken will be 10 times smaller than the actual value.

The display of an oscilloscope is arranged in a grid. The “X” axis represents time, and the “Y” axis represents amplitude. The trace/measurement of an oscilloscope travels across the screen from right to left with the right side being the most current measurement taken. When taking a measurement, the correct settings must be used to properly view the waveform. The most notable settings are voltage per division, and time per division.

When measuring DC voltage, ensure that the oscilloscope is set to measure DC, and adjust the voltage per division setting to its smallest setting while still being able to see the trace on the screen. Most oscilloscopes will provide the measured values, but it is important to know how to read what is displayed on the graph. For DC voltage the process is fairly simple, just multiply the voltage per division setting by the number of divisions that the trace is from the 0 line.

Measuring AC voltage is more complicated. Start by adjusting the time voltage per division setting to its smallest setting while still being able to see the top and bottom of the trace on the screen. Adjust the time per division setting so that you are able to clearly see at least one full cycle. Change the horizontal position so that the cycle begins on an intersection of the graph lines, this will make the calculations easier. Start by multiplying the volts per division by the number of divisions between the bottom most point to the top most point of the waveform to get the peak to peak voltage. To find the period, multiply the number of divisions between the start and end of one cycle by the time per division. And finally find the frequency by using the equation 1/period.