Inductors

The most basic form of inductor is a single straight wire. When referencing an inductor in a circuit, it is usually marked with an “L”. When an electrical current passes through the wire, a magnetic field is produced. The magnetic field’s polarity is dependent on the direction the current is flowing. A wire that is coiled will produce a stronger magnetic field than a straight wire. The more turns a coil has, the more magnetic energy can be stored.

An inductor can be compared in similar contrast with a capacitor as it behaves in a similar manner to a capacitor. An inductor stores energy in is magnetic field whenever a current is introduced similarly to how a capacitor stores energy on its plates whenever voltage is introduced. While a capacitor will resist a change in voltage, an inductor will resist a change is current. When calculating the value of multiple inductors in a circuit, the values are calculated as resistors are calculated. In parallel, inductors are calculated as Lt = product/sum. In series, the values are simply added together.

Inductance is the property of an electric conductor or circuit that causes an electromotive force to be generated by a change in the current flowing. There are many ways to increase the inductance of an inductor many of which include increasing the number of coils or the length of a coil.  If an iron or ferrite core is used in the coil, the inductance value will increase significantly without any changes to the amount of current the coil is supplied with.